CamNet User’s Guidebook

Network Camera to OBS Virtual Source

Version 2.0  |  Multiplatform (Windows + Linux)

June 2026

baldest-eagle/camnet

Table of Contents

1. Introduction	2

1.1 Key Features	2

1.2 System Architecture	3

2. Requirements and Prerequisites	3

2.1 Windows Receiver	3

2.2 Linux Receiver	3

2.3 Sender (Any Platform)	3

3. Installation	3

3.1 Windows: Virtual Camera Driver	3

3.2 Linux: V4L2 Loopback Setup	4

3.3 Python Dependencies	5

4. Quick Start Guide	7

4.1 Windows Receiver	7

4.2 Linux Receiver	8

4.3 Sender	9

4.4 Smartphone Sender	10

4.5 OBS Studio Configuration	10

4.5.1 Windows	10

4.5.2 Linux	11

5. Configuration Reference	11

5.1 Receiver CLI Options	11

5.2 Sender CLI Options	11

5.3 Shared Memory Layout	11

5.4 REST API	12

6. Platform Details	12

6.1 Windows Architecture	12

6.2 Linux Architecture	12

6.3 Platform Abstraction Layer	13

7. Troubleshooting	13

7.1 Windows: Virtual Camera Not Appearing in OBS	13

7.2 Linux: /dev/video2 Not Found	13

7.3 No Sender Devices Discovered	13

7.4 High Latency or Frame Drops	14

7.5 FFmpeg Not Found	14

7.6 Linux: Permission Denied on /dev/videoN	14

8. Advanced Usage	14

8.1 Manual Connection	15

8.2 Headless Operation	15

8.3 Multiple Cameras	16

8.4 GStreamer Hardware Encoding	16

8.5 Running Tests	16

9. File Reference	17

Right-click this table and select “Update Field” to refresh page numbers.

# 1. Introduction

CamNet is a lightweight, open-source tool that streams any network camera directly into OBS Studio as a native virtual camera device. Whether you want to use a smartphone, a secondary PC webcam, or an IP camera as a high-quality video source, CamNet makes it appear natively in OBS’s Video Capture Device list—no browser source hacks, no extra windows, no compromises. The project uses mDNS/ZeroConf for zero-configuration discovery, SRT protocol for ultra-low latency streaming, and platform-native virtual camera drivers for maximum compatibility.

As of version 2.0, CamNet supports both Windows and Linux as receiver platforms. On Windows, the receiver writes decoded frames into Win32 Named Shared Memory consumed by a DirectShow COM filter. On Linux, the receiver writes frames to a V4L2 loopback device that any V4L2-compatible application (OBS Studio, VLC, Chrome, etc.) can open as a standard video capture device. Both platforms share the same sender application and mDNS discovery protocol.

## 1.1 Key Features

| Feature | Description |
| --- | --- |
| Zero-config discovery | mDNS/ZeroConf auto-detects cameras on your LAN—no manual IP entry needed |
| Ultra-low latency | SRT streaming targets <80ms p50 on local network |
| 1080p @ 60fps | H.264 (zerolatency tune) with AAC audio passthrough |
| Audio included | Sender microphone routed alongside video automatically |
| Windows support | DirectShow COM filter appears as “CamNet Virtual Camera” in OBS |
| Linux support | V4L2 loopback device appears as “CamNet Virtual Camera” in OBS and other apps |
| REST API | Local HTTP API on port 7432 for status, control, and metrics |
| System tray | Optional system tray icon for headless operation |

## 1.2 System Architecture

CamNet follows a sender-receiver architecture. The sender device (phone, secondary PC) captures camera video, encodes it as H.264 with AAC audio, announces itself via mDNS, and streams via SRT. The receiver (primary PC) discovers senders via mDNS, ingests the SRT stream, decodes it via FFmpeg, and outputs the decoded frames to a platform-specific virtual camera. On Windows, this is a DirectShow filter reading from Win32 Named Shared Memory. On Linux, this is a V4L2 loopback device at /dev/videoN with an optional POSIX shared memory mirror for IPC consumers.

# 2. Requirements and Prerequisites

## 2.1 Windows Receiver

| Requirement | Details |
| --- | --- |
| Operating System | Windows 10 or 11 (64-bit) |
| Python | 3.11 or later |
| FFmpeg | Must be on system PATH |
| Visual Studio 2022 | Required for building the DirectShow DLL (driver build only) |
| CMake | 3.20+ for building the driver |
| Administrator access | Required for DLL registration (regsvr32) |

## 2.2 Linux Receiver

| Requirement | Details |
| --- | --- |
| Operating System | Linux kernel 5.x or later (64-bit) |
| Python | 3.10 or later |
| FFmpeg | Must be on system PATH (install via package manager) |
| v4l2loopback | Kernel module for virtual video device (DKMS package) |
| User permissions | Write access to /dev/videoN (usually requires group ‘video’) |

Tip: Automated Linux Setup Run the included linux_setup.sh script (with sudo) to automatically install v4l2loopback, FFmpeg, Python dependencies, and configure persistent module loading across reboots. The script supports Debian/Ubuntu, Fedora, and Arch Linux.

## 2.3 Sender (Any Platform)

| Requirement | Details |
| --- | --- |
| Python | 3.10 or later |
| OpenCV | opencv-python for camera capture |
| FFmpeg | Must be on system PATH for SRT streaming |
| Network | Both devices on same LAN subnet (WiFi or Ethernet) |
| Optional: GStreamer | For hardware-accelerated encoding |

# 3. Installation

## 3.1 Windows: Virtual Camera Driver

The DirectShow virtual camera driver must be installed once on the Windows receiver PC. This registers a COM DLL that OBS Studio recognizes as a video capture device. Open an elevated (Administrator) command prompt and run the installer script. The script will build the DLL using CMake and MSVC, copy it to System32, and register it with regsvr32. If you have a pre-built DLL, you can skip the build step.

py driver_installer/install.py              # Full install (build + register)

py driver_installer/install.py --skip-build # Register a pre-built DLL

py driver_installer/install.py --test-mode  # Enable test-signing (dev only)

For development, you may need to enable Windows test-signing mode (bcdedit /set testsigning on) if the DLL is not code-signed with a trusted certificate. A reboot is required after enabling test signing. For production distribution, the DLL must be signed with an EV certificate or delivered via a WHQL-signed driver package.

## 3.2 Linux: V4L2 Loopback Setup

On Linux, the virtual camera is provided by the v4l2loopback kernel module rather than a compiled driver. This module creates a /dev/videoN device that applications can read from. Install the DKMS package for your distribution and load the module with the CamNet label. The exclusive_caps=1 parameter is critical—it enables the output-only mode that v4l2loopback needs for CamNet to write frames into the device.

# Debian/Ubuntu

sudo apt install v4l2loopback-dkms ffmpeg python3-pip

sudo modprobe v4l2loopback video_nr=2 card_label='Camnet Virtual Camera' exclusive_caps=1

# Or use the automated setup script

chmod +x linux_setup.sh && sudo ./linux_setup.sh

To make the v4l2loopback module load automatically on boot, create a configuration file. This ensures the virtual camera device is available immediately after login without manual intervention. The linux_setup.sh script handles this automatically, but you can also configure it manually by writing the module name to /etc/modules-load.d/camnet.conf and the module options to /etc/modprobe.d/camnet.conf.

## 3.3 Python Dependencies

Each component has its own requirements file. On Windows, the receiver requires pywin32 for Win32 shared memory operations. On Linux, the receiver uses the standard library plus zeroconf, flask, and other cross-platform packages—no Windows-specific dependencies are needed.

# Windows receiver

cd receiver && pip install -r requirements.txt

# Linux receiver

cd receiver && pip3 install -r requirements_linux.txt

# Sender (any platform)

cd sender && pip install -r requirements.txt

# 4. Quick Start Guide

## 4.1 Windows Receiver

After installing the DirectShow driver and Python dependencies, start the receiver. It will begin listening for CamNet senders on the local network via mDNS discovery, and automatically connect to the first sender it finds. The REST API will be available at http://localhost:7432/ for monitoring and control.

cd receiver

py main.py

# With options:

py main.py --resolution 1920x1080 --fps 60 --no-auto-connect

## 4.2 Linux Receiver

After setting up v4l2loopback and installing Python dependencies, start the receiver. It will auto-detect the CamNet V4L2 loopback device by scanning /sys/class/video4linux/ for a device with the “CamNet” label. If auto-detection fails, specify the device path explicitly with the --v4l2-device option. The receiver writes decoded BGRA frames directly to the V4L2 device and optionally mirrors them to POSIX shared memory for IPC consumers.

cd receiver

python3 main.py

# Specify V4L2 device manually:

python3 main.py --v4l2-device /dev/video2

# Disable POSIX SHM mirror (V4L2-only output):

python3 main.py --disable-shm

## 4.3 Sender

On the device with the camera (secondary PC, laptop, etc.), start the sender. It will capture video from the specified device, encode it as H.264 with zerolatency tuning, announce itself via mDNS, and listen for SRT connections from the receiver. The --device flag selects the camera index (0 for default), and the --port flag sets the SRT listener port.

cd sender

py main.py --device 0 --resolution 1920x1080 --fps 60

# Without audio (video only):

py main.py --device 0 --no-audio

# Headless (no system tray):

py main.py --device 0 --no-tray

## 4.4 Smartphone Sender

For smartphone senders, no installation is required. Open the file sender/web/index.html in a browser on your phone (served from the sender PC or opened locally). The web page uses getUserMedia to access the camera and WebRTC to stream to the receiver. This is the simplest way to use a phone as a network camera source.

## 4.5 OBS Studio Configuration

### 4.5.1 Windows

In OBS Studio, add a new Video Capture Device source and select “CamNet Virtual Camera” from the device dropdown. The DirectShow filter reads frames from Win32 Named Shared Memory written by the Python receiver. For audio, add an Audio Input Capture source and select “CamNet Virtual Audio” if audio passthrough is enabled on the sender.

### 4.5.2 Linux

In OBS Studio (Linux), add a new Video Capture Device source and select “CamNet Virtual Camera” (typically /dev/video2) from the device dropdown. The V4L2 loopback device receives BGRA frames directly from the Python receiver via os.write(). For audio, configure an Audio Input Capture source pointing to the appropriate ALSA or PulseAudio device. OBS on Linux uses V4L2 natively, so the CamNet virtual camera appears just like any other webcam.

# 5. Configuration Reference

## 5.1 Receiver CLI Options

| Option | Default | Description |
| --- | --- | --- |
| --auto-connect / --no-auto-connect | True | Auto-connect to the first discovered sender |
| --resolution, -r | 1920x1080 | Capture resolution in WxH format |
| --fps, -f | 60 | Target frame rate |
| --no-tray | False | Disable system tray icon |
| --v4l2-device | auto-detect | [Linux] V4L2 loopback device path |
| --enable-shm / --disable-shm | enabled | [Linux] Enable POSIX SHM mirror alongside V4L2 |
| --verbose, -v | False | Enable verbose debug logging |

## 5.2 Sender CLI Options

| Option | Default | Description |
| --- | --- | --- |
| --device, -d | 0 | Camera device index |
| --resolution, -r | 1920x1080 | Capture resolution in WxH format |
| --fps, -f | 60 | Target frame rate |
| --port, -p | 9000 | SRT listener port |
| --no-audio | False | Disable audio capture and streaming |
| --no-tray | False | Disable system tray icon |
| --verbose, -v | False | Enable verbose debug logging |

## 5.3 Shared Memory Layout

Both Windows and Linux receivers use an identical 40-byte header followed by raw BGRA pixel data and an optional audio buffer. This layout is shared between the Win32 Named Shared Memory segment and the POSIX shared memory mirror, ensuring compatibility across platforms. The DirectShow filter on Windows and any IPC consumer on Linux can read the same binary format.

| Offset | Size | Field | Description |
| --- | --- | --- | --- |
| 0 | 4 bytes | magic | 0xCAFECAFE — identifies valid CamNet frames |
| 4 | 4 bytes | width | Frame width in pixels |
| 8 | 4 bytes | height | Frame height in pixels |
| 12 | 4 bytes | fps | Target frame rate |
| 16 | 8 bytes | frame_index | Monotonically increasing frame counter |
| 24 | 8 bytes | timestamp_ms | Milliseconds since Unix epoch |
| 32 | 4 bytes | audio_chunk_size | Bytes of audio data following pixels |
| 36 | 4 bytes | flags | Bit 0 = has_audio |
| 40 | W*H*4 | pixels | Raw BGRA pixel data |
| 40 + pixels | variable | audio | PCM s16le stereo 48 kHz audio |

## 5.4 REST API

The receiver exposes a local HTTP API on port 7432 for monitoring and external control. The /status endpoint returns the current connection state, platform information, sender details, and frame statistics. The /devices endpoint lists all discovered CamNet senders on the LAN. The /connect and /disconnect endpoints allow external tools to manage the receiver’s connection state. The /shm_info endpoint provides the shared memory layout details for C++ or Rust consumers. The /metrics endpoint returns Prometheus-style plain text metrics for monitoring systems.

| Endpoint | Method | Description |
| --- | --- | --- |
| /status | GET | Overall receiver health, stream state, platform info |
| /health | GET | Lightweight liveness check (returns 200 OK) |
| /devices | GET | List discovered CamNet sender devices |
| /connect | POST | Connect to a specific sender (body: {ip, port}) |
| /disconnect | POST | Disconnect from current stream |
| /shm_info | GET | Shared memory layout details (platform-specific) |
| /metrics | GET | Prometheus-style plain text metrics |

# 6. Platform Details

## 6.1 Windows Architecture

On Windows, the CamNet receiver writes decoded BGRA frames into a Win32 Named File Mapping (Global\CamNetFrame) protected by a Named Mutex (Global\CamNetMutex). The DirectShow COM filter (camnet_vcam.dll) opens this shared memory segment and reads frames on each FillBuffer() call. This zero-copy IPC mechanism provides sub-millisecond latency between the Python receiver and the C++ DirectShow filter. The filter appears as “CamNet Virtual Camera” in OBS’s Video Capture Device list. The driver must be registered with regsvr32 and requires Administrator privileges for installation.

## 6.2 Linux Architecture

On Linux, the CamNet receiver writes decoded BGRA frames directly to a V4L2 loopback device (typically /dev/video2) using os.write(). The v4l2loopback kernel module creates this device and makes it available to any V4L2-compatible application. OBS Studio, VLC, Chrome, Cheese, and many other Linux applications can open it as a standard video capture device. This approach requires no compiled driver—the v4l2loopback module is available as a DKMS package on most distributions.

Additionally, the Linux receiver can mirror frames to a POSIX shared memory segment (/CamNetFrame) protected by fcntl file locking. This mirror uses the exact same 40-byte header layout as the Windows shared memory, enabling cross-platform IPC consumers. The mirror is enabled by default but can be disabled with --disable-shm if only V4L2 output is needed. The V4L2LoopbackWriter class also includes auto-detection of CamNet-labeled devices by scanning /sys/class/video4linux/.

## 6.3 Platform Abstraction Layer

The receiver/platform_shm.py module provides a unified interface (FrameWriter protocol) with a factory function create_frame_writer() that returns the appropriate implementation based on sys.platform. On Windows, it returns ShmFrameWriter (from shm_writer.py). On Linux, it returns V4L2LoopbackWriter (from v4l2_output.py). Both implementations expose the same API: open(), close(), write_frame(bgra_bytes, audio_bytes), and a frame_index property. This abstraction keeps the rest of the receiver codebase platform-agnostic.

# 7. Troubleshooting

## 7.1 Windows: Virtual Camera Not Appearing in OBS

If “CamNet Virtual Camera” does not appear in OBS’s Video Capture Device list, the most common cause is that the DirectShow DLL was not registered successfully. Ensure you ran the installer script as Administrator. Check that camnet_vcam.dll exists in System32 and that the COM registration keys are present in the Windows Registry under HKCR\CLSID\{B5C7E300-6D41-4A8E-9F12-3C4D5E6F7A8B}. If you are in development mode, ensure test-signing is enabled (bcdedit /set testsigning on) and that you have rebooted.

## 7.2 Linux: /dev/video2 Not Found

If the V4L2 loopback device is not present, the v4l2loopback kernel module is likely not loaded. Run ‘sudo modprobe v4l2loopback video_nr=2 card_label=’CamNet Virtual Camera’ exclusive_caps=1’ to load it. Verify it loaded with ‘lsmod | grep v4l2loopback’ and check that /dev/video2 exists. If the module is not installed at all, install the DKMS package for your distribution (v4l2loopback-dkms on Debian/Ubuntu, v4l2loopback on Fedora). Ensure exclusive_caps=1 is set—without it, the device will not work in output mode.

## 7.3 No Sender Devices Discovered

If the receiver does not discover any sender devices, verify that both devices are on the same LAN subnet and that mDNS traffic is not blocked by a firewall. On some corporate or campus networks, mDNS (multicast DNS on port 5353) is filtered. Try disabling the firewall temporarily to test. Also ensure the sender is actually running and has successfully registered its mDNS service—check the sender’s console output for the “mDNS service registered” message.

## 7.4 High Latency or Frame Drops

If you experience high latency or frequent frame drops, first check your network conditions. CamNet targets <80ms p50 latency on a local network, but WiFi congestion or poor signal can significantly increase this. For best results, use a wired Ethernet connection for at least the receiver. On the sender side, ensure the camera resolution and FPS are not exceeding what the device can encode in real-time. You can reduce the resolution or FPS to lower the encoding burden. Also verify that FFmpeg is using the zerolatency tune and ultrafast preset.

## 7.5 FFmpeg Not Found

Both the sender and receiver depend on FFmpeg being available on the system PATH. On Windows, download FFmpeg from the official site and add its bin directory to your PATH environment variable. On Linux, install it via your package manager (apt install ffmpeg, dnf install ffmpeg, or pacman -S ffmpeg). Verify the installation by running ‘ffmpeg -version’ in a terminal.

## 7.6 Linux: Permission Denied on /dev/videoN

If you get a permission error when writing to the V4L2 device, your user account likely does not have the necessary permissions. Add your user to the ‘video’ group with ‘sudo usermod -aG video $USER’, then log out and log back in. Alternatively, run the receiver with sudo (not recommended for regular use). You can also check the device permissions with ‘ls -la /dev/video2’ and adjust the udev rules if needed.

# 8. Advanced Usage

## 8.1 Manual Connection

By default, the receiver auto-connects to the first discovered sender. To manually choose which sender to connect to, start the receiver with --no-auto-connect and use the REST API to initiate the connection. First, query the /devices endpoint to list available senders with their IP addresses and ports, then send a POST request to /connect with the desired sender’s IP and port in the request body.

py main.py --no-auto-connect

# Then connect via API:

curl -X POST http://localhost:7432/connect -H 'Content-Type: application/json' \

-d '{"ip": "192.168.1.100", "port": 9000, "device_name": "my-laptop"}'

## 8.2 Headless Operation

For server or headless environments where no desktop is available, use the --no-tray flag to disable the system tray icon. The receiver will run entirely in the console, and all control is done via the REST API or keyboard interrupt (Ctrl+C to stop). This is useful for running the receiver as a background service or in a Docker container.

## 8.3 Multiple Cameras

Currently, the receiver supports one active stream at a time. To use multiple cameras simultaneously, you would need to run multiple receiver instances on different SRT ports with different virtual camera devices. On Linux, you can create multiple v4l2loopback devices (e.g., video_nr=2,3,4) and run separate receiver processes, each writing to a different /dev/videoN device. On Windows, each receiver instance would need its own DirectShow filter instance with a unique shared memory name.

## 8.4 GStreamer Hardware Encoding

The sender supports GStreamer as an alternative encoding backend for hardware-accelerated H.264 encoding. On Windows, this uses DXVA2 or D3D11; on macOS, VideoToolbox; on Linux, V4L2 M2M or VA-API. The GStreamerEncoder class in sender/encoder.py auto-detects available hardware encoders (mfxh264enc for Intel Quick Sync, nvh264enc for NVIDIA NVENC, amfh264enc for AMD AMF, vtenc_h264 for Apple VideoToolbox) and falls back to software x264 if none are available. To use GStreamer, install the GStreamer runtime and Python bindings (gi).

## 8.5 Running Tests

The project includes a test suite covering mDNS discovery round-trips, frame latency benchmarks, shared memory read/write correctness, and network dropout recovery. Run the full test suite with pytest from the project root directory.

py -m pytest tests/ -v --tb=short

# Individual benchmarks:

py tests/test_latency.py --duration 30 --target-p99 150

py tests/test_shm.py

# 9. File Reference

| Path | Description |
| --- | --- |
| sender/main.py | Sender entry point, CLI, system tray |
| sender/capture.py | Camera capture (OpenCV + GStreamer backends) |
| sender/encoder.py | H.264 + AAC encoding (FFmpeg + GStreamer) |
| sender/streamer.py | SRT server-mode streamer + WebRTC fallback |
| sender/discovery.py | mDNS service announcer (zeroconf) |
| sender/web/ | Browser-based sender for smartphones |
| receiver/main.py | Receiver entry point (Windows + Linux) |
| receiver/discovery.py | mDNS service browser + device registry |
| receiver/ingest.py | SRT stream ingest + FFmpeg decode |
| receiver/shm_writer.py | Win32 Named Shared Memory writer (Windows only) |
| receiver/v4l2_output.py | V4L2 loopback writer + POSIX SHM mirror (Linux only) |
| receiver/platform_shm.py | Platform-abstracted FrameWriter factory |
| receiver/controller.py | REST API controller (Flask) |
| driver/ | DirectShow COM filter C++ source (Windows only) |
| driver_installer/ | DLL build + COM registration scripts (Windows only) |
| linux_setup.sh | Automated Linux setup script (v4l2loopback, ffmpeg, deps) |
| tests/ | Test suite (discovery, latency, dropout, SHM) |